Report: Transforming Raw Gyroscope Data into Behavioral Rotation Features

1. Introduction

The gyroscope sensor measures the phone’s angular velocity along three axes (x, y, z). Unlike accelerometer data—which captures physical movement—gyroscope signals capture rotation, orientation changes, shaking, and phone handling.

Raw gyroscope logs provide hundreds or thousands of detailed readings per minute, but this data is too large and too noisy to use directly.
Therefore, we convert raw gyroscope data into compact behavioral features and assign rotation labels that represent the user’s activity level.

2. Raw Data Structure

The Beiwe gyroscope logs contain:

- timestamp (ms)

- UTC time

- x-axis rotation

- y-axis rotation

- z-axis rotation

Example raw row:

timestamp: 1647832267533

x = 0.162

y = 0.495

z = -0.391

These values represent angular velocity, not linear acceleration.

3. Computing Rotation Magnitude

To convert the three rotation axes into a single interpretable value, we compute:

gyro_mag=x2+y2+z2

This gives one continuous number describing how fast the phone is rotating.

Example:

x = 0.534, y = 0.191, z = -0.879

gyro_mag ≈ 1.03   (strong rotation)

4. Resampling and Feature Extraction

Raw gyro data arrives at ~10–100 samples per second.
To reduce dataset size while preserving meaning, we group data into 1-second windows and compute:

- mean_gyro → avg rotation strength

- std_gyro → jitter (stress or shaky movement)

- max_gyro → strongest rotation spike

This reduces:

- ~100 rows per second
→ 1 row per second

Massive compression while keeping useful behavioral patterns.

5. Rotation Label Classification

To make the features practically useful, we classify each time window into one of four categories:

1. no_rotation

- mean_gyro < 0.1

- The phone is still or resting on a surface

- No noticeable rotation

2. slight_rotation

- mean_gyro between 0.1 and 0.5

- Small hand adjustments

- Low-level phone movement

- Typical when holding the phone gently

3. phone_handling

- mean_gyro between 0.5 and 1.5

- Normal phone usage

- Typing, scrolling, repositioning the phone

- Not heavy rotation but clearly active handling

4. high_rotation

Triggered when:

- mean_gyro > 1.5
OR

- max_gyro > 2.0

High rotation represents:

- vigorous phone movement

- rapid twisting

- shaking

- sudden orientation changes

- stressed / restless motion

This category detects strong bursts of rotational activity.

6. Example Output From Your Gyro File

Your processed file produced:

datetime  mean_gyro  std_gyro  max_gyro   rotation_label

0  2022-03-21 01:31:07   0.662911  0.242737  1.046909   phone_handling

1  2022-03-21 01:31:08   0.305198  0.178471  0.651508  slight_rotation

2  2022-03-21 01:31:09   0.647967  0.421930  1.544262   phone_handling

3  2022-03-21 01:31:10   0.284260  0.212140  0.799819  slight_rotation

4  2022-03-21 01:31:11   0.320891  0.207788  0.683312  slight_rotation

5  2022-03-21 01:31:12   0.203576  0.127260  0.481797  slight_rotation

6  2022-03-21 01:31:13   0.443351  0.926322  3.067789    high_rotation

7  2022-03-21 01:31:14   0.623965  0.574033  2.148466    high_rotation

8  2022-03-21 01:31:15   1.565925  0.778303  2.760208    high_rotation

9  2022-03-21 01:31:16   0.216598  0.294728  0.798186  slight_rotation

7. How We Concluded Each Rotation Label (Explained Clearly)

Row 0

mean_gyro = 0.66  → between 0.5 – 1.5

max_gyro  = 1.04  → low spike

Label → phone_handling

This means the user is actively holding/using the phone.

Row 1

mean_gyro = 0.30 → slight rotation range

Label → slight_rotation

Small movements, low intensity.

Row 2

mean_gyro = 0.64 → phone_handling range

max_gyro = 1.54 → high enough to confirm handling

Label → phone_handling

Row 6

mean_gyro = 0.44 → looks normal

BUT max_gyro = 3.06 → very strong spike

Label → high_rotation

This means there was brief strong twisting/shaking.

Row 8

mean_gyro = 1.56 → above 1.5 threshold

max_gyro = 2.76 → also very high

Label → high_rotation

This indicates:

- strong rotation

- sudden motion

- phone moved quickly or forcefully