Multimodal Smartwatch Behavioral Signature Modeling for Mood Episode and Relapse Risk Prediction

1. Introduction

Mood disorders such as major depressive disorder (MDD) and bipolar disorder (BD) are characterized by disturbances in autonomic regulation, sleep–wake rhythms, circadian phase alignment, and behavioral activity patterns. Traditional assessment relies on interviews and self-report questionnaires, which are subject to recall bias and delayed detection [fpsyt-16-1695158.pdf, L67–73].

Wearable devices enable continuous passive collection of physiological and behavioral data that may function as digital biomarkers [fpsyt-16-1695158.pdf, L21–24].

Heart rate variability (HRV) represents a non-invasive measure of autonomic nervous system (ANS) regulation [fpsyt-16-1695158.pdf, L98–102], and reductions in HRV have been associated with depression and anxiety disorders [fpsyt-16-1695158.pdf, L113–120].

Multimodal biosignal analysis enhances reliability and reduces false positives compared to single-signal detection [s43856-025-01234-6.pdf, L1–6; L14–15].

This chapter describes the full mathematical, computational, and architectural framework used in our system to detect smartwatch-derived behavioral signatures associated with mood destabilization and relapse risk.

2. System Architecture Overview

2.1 High-Level Architecture

Figure 1 (Insert Diagram): System Architecture Overview

Smartphone Sensors      Wearable Sensors
        │                       │
        ├──────────┐   ┌────────┤
        │          │   │        │
Hourly Feature  Daily Feature Extraction
Extraction           │
        │            │
   LSTM Autoencoder  │
        │            │
   Hourly Anomalies  │
        │            │
        └────── Fusion Layer ──────┐
                                    │
                           Signature Engine
                                    │
                           Risk Score Output

2.2 Sensor Streams Used

Smartphone Sensors (Hourly Resolution)

- Accelerometer (tri-axial)

- Gyroscope

- GPS location

- Screen events

- Notifications (optional behavioral proxy)

- Battery (data integrity)

Wearable Sensors (Daily Resolution)

- Step counter

- Sleep episodes

- Heart rate (PPG-derived)

- Oxygen saturation

- Respiratory rate

- Blood pressure (optional)

- Weight (long-term health covariate)

3. Mathematical Signal Processing

3.1 Accelerometer Signal Modeling

Raw vector:

at=(xt,yt,zt)

Magnitude:

At=xt2+yt2+zt2

Jerk:

Jt=∣At-At-1∣

Hourly features:

μA=1N∑At
σA=1N-1∑(At-μA)2

Inactivity index:

I=#(At<P10)N

Where P10= 10th percentile threshold.

Interpretation:

- High inactivity + low GPS mobility may indicate withdrawal.

3.2 GPS Mobility Modeling

Total distance:

D=i=2nhaversine(Pi-1,Pi)

Stationary ratio:

S=#(di<5m)n

Low mobility has been linked to depressive behavioral withdrawal in digital phenotyping literature.

3.3 Sleep Modeling and Circadian Features

Sleep windows are constructed using 10-minute resolution and grouped within 1-hour boundaries [s41746-024-01333-z.pdf, L367–372].

Sleep duration:

SleepMint=episodesendstart60,000

Sleep midpoint:

Midpoint=start+duration2

Circadian phase modeling may approximate DLMO by subtracting 7 hours from CBTmin [s41746-024-01333-z.pdf, L152–154].

Variability measure:

SD7d(SleepStart)

Circadian rhythm amplitude represents rhythm strength [s41746-024-01333-z.pdf, L150–153].

3.4 Heart Rate and HRV Modeling

HRV is derived from RR intervals and categorized into time-domain and frequency-domain measures [fpsyt-16-1695158.pdf, L98–102].

Time-Domain Metrics

RMSSD=1N-1∑(RRi+1-RRi)2
SDNN=1N-1∑(RRi-RR)2

RMSSD reduction precedes depressive symptom exacerbation [fpsyt-16-1695158.pdf, L29–31].

Abnormal HRV days defined as exceeding ±1.5 SD of personal mean [fpsyt-16-1695158.pdf, L235–237].

Frequency-Domain Metrics

LF power, HF power, LF/HF ratio reflect sympathetic–parasympathetic balance [s43856-025-01234-6.pdf, L31–34].

LF/HF ratio indicates sympathetic dominance during anxiety states [s43856-025-01234-6.pdf, L31–33].

3.5 Electrodermal Activity (Future Extension)

EDA reflects sympathetic nervous system activation [s43856-025-01234-6.pdf, L39–42].

Multimodal combinations improve accuracy [s43856-025-01234-6.pdf, L14–15].

4. Baseline Modeling and Anomaly Detection

Within-person normalization is recommended over population thresholds [smartwatch_behavioral_signatures.pdf, L63–64].

Standard z-score:

z=x-μσ

Robust z-score:

zr=0.6745⋅x-medianMAD

Anomaly threshold example:

∣z∣>2

[smartwatch_behavioral_signatures.pdf, L72–73]

5. Signature Construction Framework

Each signature is a composite function:

Sk(t)=i=1mwizi,t

Where:

- zi,t= baseline-normalized feature

- wi= learned weight

6. Detailed Signature Definitions

WD1 — Reduced Activity

Sensors: wearable_steps
Trigger condition:

zsteps≤-2

for ≥ 2 days.

WD2 — Sleep Disruption + HR Trend

Sensors: wearable_sleep + wearable_heart_rate

Trigger:

zsleep≤-2∧znightHR≥2

HRV reductions linked to depression [fpsyt-16-1695158.pdf, L29–31].

WB1 — Activity Variability + Sleep Irregularity

Sensors: steps + sleep timing

SD7d(Steps)>1.7×SDbaseline

Sleep-wake indexes modeled with SD and Z-scores at individual level [s41746-024-01333-z.pdf, L159–160].

WB2 — Elevated Activation Pattern

Trigger if:

zsleep≤-2
zsteps≥2
znightHR≥2

Multi-signal confirmation preferred [smartwatch_behavioral_signatures.pdf, L61–67].

WA1 — Physiological Arousal

Sensors: HR (PPG), future EDA/respiratory

HR and HRV are widely used anxiety indicators [s43856-025-01234-6.pdf, L11–16].

WA2 — Night Restlessness

Sleep fragmentation modeling supported by window segmentation [s41746-024-01333-z.pdf, L367–372].

WSU3 — Baseline Relative Anomaly Rate

AnomalyCount=∑1[∣zi∣>2]

Trigger if:

AnomalyCount≥2

7. Deep Learning Component — LSTM Autoencoder

Figure 2 (Insert Diagram): LSTM Autoencoder Architecture

Input Sequence (24 hours × F features)
        ↓
LSTM Encoder
        ↓
Latent Vector
        ↓
LSTM Decoder
        ↓
Reconstructed Sequence

Reconstruction loss:

E=1TF∑(X-X)2

Anomaly:

E>μE+2σE

Consistent with anomaly-based detection framework [smartwatch_behavioral_signatures.pdf, L72–73].

8. Multimodal Fusion

Multimodal wearable detection outperforms single-signal detection [s43856-025-01234-6.pdf, L14–15].

Fusion model:

Risk(t)=σ(β1SWD1+β2SWD2+⋯+β7SWSU3)

Sigmoid:

σ(x)=11+e-x

9. Model Evaluation Framework

Wearable AI pooled performance values reported up to ~0.89 accuracy [s41746-023-00828-5.pdf, L8–12].

Metrics:

- AUC

- Sensitivity

- Specificity

Cross-validation recommended due to limited wearable dataset sizes [s41746-023-00828-5.pdf, L26–34].

10. Limitations and Research Roadmap

Missing RR intervals → cannot compute true HRV

HRV requires RR intervals [fpsyt-16-1695158.pdf, L98–102].

Missing EDA → anxiety detection weaker

EDA directly reflects SNS activity [s43856-025-01234-6.pdf, L39–42].

No TAC sensor → cannot implement WSU2

No light exposure modeling → circadian modeling limited

Circadian amplitude modeling supported in literature [s41746-024-01333-z.pdf, L150–153].

11. Future Architecture Diagram

Figure 3 (Insert Diagram): Future Enhanced Sensor Architecture

Add:

- RR interval API

- EDA device

- Light exposure sensor

- TAC device

12. Conclusion

Our multimodal smartwatch behavioral signature model:

- Extracts validated physiological features.

- Applies within-person baseline normalization.

- Detects multistream anomaly clusters.

- Uses deep sequence modeling (LSTM-AE).

- Implements privacy-preserving aggregation.

- Aligns with current wearable AI evidence [s41746-023-00828-5.pdf, L8–12].

- Incorporates circadian and HRV research foundations [fpsyt-16-1695158.pdf, L29–31; s41746-024-01333-z.pdf, L150–160].

13. References

https://www.nature.com/articles/s41746-024-01333-z

https://www.nature.com/articles/s41746-023-00828-5

https://medinform.jmir.org/2025/1/e66277/

https://www.frontiersin.org/journals/psychiatry/articles/10.3389/fpsyt.2025.1695158/full

https://www.nature.com/articles/s43856-025-01234-6

https://onlinelibrary.wiley.com/doi/abs/10.1111/acps.13765